# 5G Standalone Network

This lab follows four user devices through registration and session setup, then compares two example QoS flows competing for a shared radio-side service queue. You will see the roles of the gNB, AMF, SMF, and UPF, how queue scheduling affects traffic, and how OMNeT++ records results over time.

# Learning goals

Trace registration and PDU-session setup through 5G core functions.

Trace user packets through the gNB scheduler, UPF, and data network.

Explain the effect of strict-priority scheduling and a finite queue.

Compare control-plane setup delay with user-plane packet delay.

Read scalar and vector output for packet delivery, queueing, delay, and throughput.

# Topology

2 URLLC-profile UEs ─┐                         ┌─ AMF ── SMF
                       ├── gNB ── N3 ── UPF ─────┤        │
   2 eMBB-profile UEs ─┘           shared RAN    │        N4
                                  queue           │        │
                                                   └────── Data Network

The drawing is conceptual: the AMF and SMF exchange control messages; the SMF configures UPF session state over the modeled N4 link. The gNB forwards UE control messages to the AMF and schedules user traffic over a single shared service queue.

# What the model represents

| Component | Role in this lab |
| --- | --- |
| UE | Sends a registration request, requests a PDU session, then generates user packets. |
| gNB | Routes control messages and models a rate-limited radio-side queue with a priority flow and finite waiting buffer. |
| AMF | Accepts UE registration and forwards session requests to the SMF. |
| SMF | Sends session-install messages to the UPF and accepts the session after the UPF replies. |
| UPF | Checks each packet against the installed UE session and forwards matching user traffic. |
| Data network | Receives packets and records delay, throughput, and packet-delivery results. |

Messages are simplified OMNeT++ objects, not 3GPP wire protocol messages. The URLLC/eMBB names are example traffic profiles. QFI identifies a QoS flow; it is not a network-slice identifier. The model does not implement slice isolation, 5QI mapping, a physical radio channel, mobility, retransmissions, handover, security, or a full 3GPP QoS scheduler. The priority rule is an explicit teaching abstraction: QFI 7 is treated as high priority at the gNB.

# Follow registration and session setup

Each UE sends REG_REQ through the gNB to the AMF.

The AMF returns REG_ACCEPT through the gNB. The UE records registration-response time.

The UE sends PDU_REQ; the AMF forwards it to the SMF.

The SMF sends N4_INSTALL to the UPF with the UE, flow label, and QFI.

The UPF stores the session and replies N4_ACK. The SMF and AMF return PDU_ACCEPT to the UE.

Only after acceptance does the UE start sending user-plane packets.

Control-plane setup delays affect when traffic can begin. After setup, user packets use the separate data path: UE → gNB queue → UPF → data network.

# Traffic and scheduler settings

| Profile | Number of UEs | QFI | Packets per UE | Packet size | Interval |
| --- | --- | --- | --- | --- | --- |
| URLLC example | 2 | 7 | 40 | 128 bytes | 10 ms |
| eMBB example | 2 | 9 | 30 | 1200 bytes | 15 ms |

The default gNB service rate is 5 Mbps. It serves one packet at a time and selects a waiting QFI 7 packet before a QFI 9 packet. The waiting queue holds at most 160 packets in the default experiment. When full, an arriving high-priority packet can displace a queued eMBB packet; a low-priority packet is dropped if no room can be made. An in-service packet is not interrupted.

# Run the project

Open 5G_OMNeT_Lab as an OMNeT++ 6.4 project.

Build the project and open omnetpp.ini.

Choose a configuration and start Qtenv to observe events or Cmdenv to read the log.

To build from the OMNeT++ shell, run opp_makemake -f --deep -o fiveglab, then make MODE=release all.

A command-line run looks like ./out/clang-release/fiveglab.exe -u Cmdenv -f omnetpp.ini -n . -c HighLoad.

The four configurations are:

| Configuration | Change from default | Question it helps answer |
| --- | --- | --- |
| General | 5 Mbps service rate, moderate traffic, 160 waiting slots | What are the baseline results with little congestion? |
| FastControlPlane | 1 ms control and core links instead of 5 ms and 2 ms | How does faster signaling affect setup time? |
| LowRANCapacity | gNB service rate reduced to 1.5 Mbps | How does a tighter shared radio resource affect delay? |
| HighLoad | 400 offered packets and a 40-packet waiting queue | Which flow experiences queueing and loss under overload? |

All traffic and link values are configured in omnetpp.ini and 5GLab.ned.

# Recorded experiment results

These are measured results from the included OMNeT++ 6.4 scalar files. See Results.md for the complete tables and file names.

| Configuration | Offered / received | Delivery | URLLC delay | eMBB delay | Key observation |
| --- | --- | --- | --- | --- | --- |
| General | 140 / 140 | 100% | 8.478 ms | 8.930 ms | No drops; maximum waiting queue was 2. |
| FastControlPlane | 140 / 140 | 100% | 8.478 ms | 8.930 ms | PDU-session setup fell from 22 ms to 10 ms. |
| LowRANCapacity | 140 / 140 | 100% | 11.915 ms | 14.881 ms | Delay and queue wait increased as service rate fell. |
| HighLoad | 400 / 363 | 90.75% | 7.937 ms | 61.496 ms | 37 eMBB drops; no URLLC drops; eMBB queue wait averaged 52.576 ms. |

The measured results follow the configured policy: the gNB serves priority packets first. The URLLC example keeps low queue wait under load, while eMBB experiences a longer queue and packet loss. These values are outputs of this simplified model; they are not predicted real-network performance.

The fast-control experiment is useful for distinguishing setup and data-plane delay. Registration response averaged 14 ms in General and 6 ms in FastControlPlane; PDU-session setup averaged 22 ms and 10 ms. Packet delay after session setup stayed about the same because the data path did not change.

# Read scalar and vector results

Each run writes a .sca file for one-value summaries and a .vec file for timestamped samples under results/.

UE scalars record packetsSent, registrationDelay_ms, and pduSessionSetupDelay_ms.

gNB scalars record queue sizes, mean wait by class, packets served, and drops.

Data-network scalars record sent and received totals, delivery ratio, mean delay, bytes, throughput, and per-class values.

Vectors record queue length by class, queue wait per packet, gNB drops and service count, per-packet end-to-end delay, and cumulative bytes received.

In OMNeT++ Result Analysis, plot gnb.ranQueueLength_packets with gnb.urlQueueLength_packets and gnb.embbQueueLength_packets to compare queue occupancy. Plot dataNetwork.urlEndToEndDelay_ms and dataNetwork.embbEndToEndDelay_ms to compare delay samples. Plot dataNetwork.cumulativeReceivedBytes against simulation time; the slope estimates received throughput.

Packet delivery ratio is:

packets received at data network / packets sent by all UEs × 100%

Throughput over a selected window is:

received bytes × 8 / elapsed seconds

The .sca file also records these summary values automatically.

# Lab exercises

Reduce *.gnb.ranDataRate from 5 Mbps to 2 Mbps. Compare per-class queue waits and delay.

In HighLoad, raise *.gnb.maxQueuePackets from 40 to 100. How do eMBB drops and delay change?

Set *.gnb.highPriorityQfi = 9. Which flow gets priority now? Compare drops and mean queue wait.

Change FastControlPlane's control-link delay to 10 ms. Predict and measure registration and PDU-session setup time.

Double one UE's packet count without changing its interval. Compare offered load, delivery ratio, and queue length.

Change processingDelay at the UPF. Which measurements should change, and which should remain nearly constant?

# Discussion questions

Which network function handles registration? Which establishes the session? Which forwards user traffic?

Why does the UE wait for PDU-session acceptance before sending packets?

What is the difference between the gNB's queueing time and end-to-end delay?

Why does reducing control-plane delay change setup time but not the delay of packets sent afterward?

Why does strict priority protect the configured QFI under overload? What trade-offs does it create for eMBB?

Which parts of the model would need to change to study mobility, radio retransmissions, or real slice isolation?

# Source files and references

5GLab.ned describes the topology and parameterized modules.

5GLab.cc implements UE setup, gNB queueing, UPF session checks, and metrics.

omnetpp.ini defines the experiments and result output.

3GPP TS 23.501: System architecture for the 5G System.

OMNeT++ 6.4 simulation result collection.